Sony CM-DX1000 / Siemens S4 connector

view :  from lower phone side.

pin function:

1.	Power ( + 9 volt )

2.	Battery charge

3.*	Handsfree sos

4.	Handsfree extern  (10K resistor to +9 to enable handsfree functions)

5.*	TXE

6.*	RXE

7.	Ignition (10K resistor to +9 to illuminate the display continiously)

8.	Antenna extern (10K resistor to ground to enable external microphone and antenna)

9.*	Audio 1

10.	Audio 2  (loudspeaker amplifier output)

11.	Microphone (use electrect microfoon and  46 db preamp.)

12.	Audio ground  (do not connect to 13 and 14 )

13.	Ground (supply)

14.	Ground (supply)

(*) Means not sure

Way to find out about any other connector pinlayout

The dutch firm BMS electronics ( Postbox 633    6130AP Sittard Holland ) supplies universal handfreekits.

To use these kits for a certain phone you should buy a interface cable between the phone and the handsfreeset.

The trick to this is that any phone connector is interfaced to a six pin modular phone connector such as used in normal wall oulets for normal phones.

So when you buy a cable for your own phone and open the phone connector and folow the wires to the pins you can determan wich pin has wich function acording to the color of the wires.

The wires in these interface cable have the folowing functions:

White	: Audio

Black	: Microphone

Red	: Audio ground

Green	: Ground (supply)

Yellow	: not used for the phone only to tell the HF set the cable is attached to it

black	: supply voltage (phone dependent)

copyright 1997 Ton Colen (ton.colen@dsv.nl)